ORPHEUS AUDIO PLAYER USER MANUAL

INTRODUCTION

Welcome to the Orpheus Audio Player System—a fully assembled, ruggedized solution for scheduled wildlife audio playback. Orpheus was originally developed for ornithological research in demanding field environments, with all hardware and software components engineered to function together seamlessly. This manual provides guidance on operating and maintaining the device, along with important information on lawful and ethical usage.

LEGAL AND ETHICAL USE

Orpheus is intended for research, educational, and lawful observation of wildlife. Some jurisdictions regulate or prohibit the use of recorded calls to attract, capture, or hunt animals. Users are solely responsible for ensuring they comply with all applicable local, regional, and national laws regarding wildlife protection, conservation, and hunting. This may include:

- Obtaining any necessary permits or licenses

- Adhering to restrictions on using audio lures for disturbing or capturing protected species

- Following ethical guidelines to avoid undue stress on wildlife

GaiaForge Technology assumes no liability for damages, fines, or legal consequences resulting from misuse or unauthorized use of this device. By operating the Orpheus Audio Player, you acknowledge responsibility for its deployment and agree to abide by relevant laws.

SYSTEM OVERVIEW

Orpheus is delivered as a fully integrated unit featuring all core hardware and software:

- A built-in Raspberry Pi Zero 2 W serving as the primary processor

- A Class D audio amplifier and speaker system

- A high-capacity battery pack (with solar-charging)

- An integrated microcontroller for advanced power management

- Weatherproof enclosure and connectors to ensure reliable operation in challenging environments

This design supports multiple playback modes and advanced features without requiring any additional assembly. Orpheus’s main functionalities include:

- Manual Playback: Direct user control over audio, (Pro unit includes PA mode and Bluetooth connection capability)

- Interval Playback: Time-based scheduling

- Astral Playback: Scheduling based on local sunrise/sunset

- Seasonal Playback: Timing aligned to natural or custom seasons

- Recording Mode: Capturing environmental audio for analysis (Pro unit only)

- Analytics: Tracking and exporting playback data (logging only available on pro unit)

- Power Management: Optimizing battery usage via smart sleep/wake cycles

GETTING STARTED

Unboxing and Setup

- Inspect the Device: Remove Orpheus from the packaging and ensure all components (enclosure, display/touch interface if included, speaker connections) are intact.

- Power Connection: Connect the provided LiFePo 4 battery using the XT60 connector. Connect the MC4 solar connectors observing polarity.

- Location Data: On first boot, Orpheus will prompt you to enter location details (GPS coordinates) under Settings → Astral Setup for correct sunrise/sunset calculations.

- Speaker Check: Connect the speakers to the waterproof connectors.  Confirm audio output is functional by performing a brief test using manual mode and playing the test tone.

Orpheus comes preloaded with all necessary software—no image flashing or DIY assembly is required.

MAIN INTERFACE

- Mode Selector: A dropdown or touchscreen menu offering Manual, Interval, Astral, Seasonal, or Recording modes.

- Toolbar: Quick access to System Settings, File Management, Analytics, and Power options.

- Status Area: Displays current date/time and active playback status.

- Main Panel: Changes based on the mode selected.

PLAYBACK MODES

1. Manual Playback

- Select Load Audio to choose a file.

- Use Play, Pause, or Stop buttons.

- Track information appears in the Status Area.

2. Interval Playback

- Select Set Interval to create a time-based schedule (daily, weekdays, weekends, or custom).

- Set start/end times and assign audio files.

- Orpheus automatically plays audio during those intervals.

3. Astral Playback

- Under Set Astral Interval, choose Sunrise or Sunset as your baseline.

- Define how many hours before/after these times playback should occur.

- Orpheus recalculates sunrise/sunset daily from your GPS coordinates.

4. Seasonal Playback

- Choose Natural Seasons (Spring, Summer, Fall, Winter) or select Custom Seasons via the checkbox to define date ranges and create playlists.

- Assign audio files or intervals to each season.

- Particularly useful for matching migration patterns or other seasonal wildlife behaviors.

5. Recording Mode

- Choose Storage: Orpheus includes onboard storage; you may also connect external storage if applicable.

- Adjust Audio Quality: Select desired bitrate or bit depth.

- Recording Scheduling: Allows user to schedule when recording should begin and end.

- Start Recording: Captures environmental sounds; status area shows elapsed time.

- Stop Recording: Finalizes and saves the audio file.

SYSTEM SETTINGS

Audio File Management

- Access via the "Audio" icon or from the main menu.

- Review, delete and manage files stored on Orpheus.

- USB Import: Connect a USB flash drive to add audio tracks.

- USB Export: Uploads audio tracks from the device to a USB flash drive.

Astral Setup

- Enter or confirm GPS coordinates and local timezone.

- Define a device Nickname e.g.  “Zone 1”

Time/Date

- Preset at factory to Real Time Clock (battery life 3-6 years).

- Otherwise, manually set the system clock as needed which will sync the RTC time if battery is depleted.

GPIO Settings

- The GPIO controls the Amplifier power. Can be disabled by the user. Only disable if you are using an external amplifier.

Analytics

- Enable Logging: Store data on playback events.

- Filter Logs: By date.

- Export: Save logs to USB or double click for a preview on the HMI.

POWER MANAGEMENT

Basic Power Management

- Orpheus can automatically power down between scheduled events.

- It wakes up shortly before the next playback.

- A manual wake button or remote command can bring the system online at any time.

Advanced Power Management

- The Power Management module oversees deep sleep, drastically reducing power consumption.

- The main processor signals the next wake time to the Microcontroller, then fully shuts down.

- The ESP32 reactivates the system before a scheduled event.

FIELD DEPLOYMENT TIPS

- Battery Life: Longer intervals between playback events yield greater battery savings. Monitor usage and logs initially.

- Weather Protection: Orpheus ships in a weather-sealed enclosure; ensure connectors remain properly sealed.

- Data Collection: Regularly export logs for analysis if research or compliance requirements apply.

- Remote Access: Enable Wi-Fi or the built-in access point for the companion app if desired.

TROUBLESHOOTING

System Won’t Power On

- Check the battery connection and press the power switch.

- Press the Manual Wake button.

- If the system still will not start, press the master power button.

- Contact support if none of the methods work

No Audio Output

- Confirm GPIO is on under System Settings → GPIO settings.

- Verify volume levels and the selected audio file format.

- Ensure Mute is not activated in the current playback mode

Scheduled Playback Not Triggering

- Ensure correct system time and location data.

- Check interval or astral/seasonal configuration.

- Ensure audio file is not corrupted and is either WAV or MP3 format

Recording Issues

- Confirm sufficient storage space.

- Check microphone or external audio input connections (pro unit only).

MAINTENANCE

Software Updates

- Connect Orpheus to a network.

- Go to Settings → Check for Updates.

- Follow on-screen instructions.

File Management

- Regularly remove unused audio or log files to free space.

- Backup important configurations.

LEGAL COMPLIANCE & DISCLAIMER

- Local Laws: Use of recorded calls for wildlife attraction, capture, or hunting may be restricted or prohibited. Obtain all necessary permits.

- User Liability: You are responsible for complying with environmental, conservation, and hunting regulations. GaiaForge Technology cannot be held liable for misuse.

- Ethical Operation: High-volume or constant playback may disturb wildlife. Use responsibly to minimize ecological impact.

Important: If you are conducting scientific research, confirm any additional requirements or institutional approvals.

SUPPORT

- For advanced configuration, consult the technical documentation provided.

- Visit contact@gaiaforge.tech for additional help.

- For questions on legal usage, contact local authorities or your institution’s compliance office.

© 2024 GaiaForge Technology. All rights reserved.