Orpheus Pro - Technical Documentation File

Device Overview

Product Name: Orpheus Pro Bio-Acoustic Playback Device
Application: Bird acoustic playback, PA integration, Bluetooth playback, environmental monitoring, data logging
Target Market: Research, wildlife management, ornithology

Key Features vs Basic Version:

- Advanced data logging capabilities

- PA system integration

- Bluetooth audio playback

- Environmental monitoring (temperature, humidity, pressure)

- Recording mode capability (microphone TBD)

- Victron SmartSolar MPPT with Bluetooth monitoring

- Enhanced power management and monitoring

Complete Component Specifications

Power Management System

- Battery: 4x LiFePO4 cells (IFR32700, 3.2V, 6000mAh each = 12.8V, 6Ah system)

- BMS: JBD CP04S005 (4S, 10A smart BMS with passive balancing)

- Solar Charging: BQ24650 MPPT Solar Controller + 30W solar panel

- Power Distribution:

  - Pololu 5V 2.5A step-down converter (Model D24V25F5) - Raspberry Pi power

  - Pololu 500mA DC/DC step-down converter - ESP32 power

- Power Monitoring: INA219 power sensor

- Power Switch: Pololu Mini Pushbutton Power Switch LV

Processing & Communication

- Main Controller: Raspberry Pi Zero 2W (WiFi capability)

- Secondary Controller: ESP32 WROOM (BLE capability)

- ESP32 Interface: ESP32 Breakout Board (3.5mm terminal GPIO expansion)

- Real-Time Clock: DS3231 RTC module

- Environmental Sensor: Bosch BMP280 (temperature, humidity, pressure)

- Interconnect: 40-pin GPIO cable (15cm)

Audio System

- Audio Interface: IQaudio Zero Codec Hat

- Ground Loop Isolator: Audio ground loop isolator (codec to amplifier isolation)

- Amplifier: Sure AA-TPA3116D2 Class-D Amplifier (2-channel, 50W, TPA3116 chip)

- Speakers: Herdio outdoor 4-ohm waterproof speakers

- Audio Connections: Waterproof speaker connectors

Control & Interface

- Display: 5" Capacitive Touch Screen LCD (800×480)

- Relay Control: 3.3V opto-isolated 1-channel relay

Enclosure & Environmental

- Enclosure: IP65 rated (21 x 28 x 13 cm)

- Cable Management: M10 cable glands

- Mounting: Nylon standoffs (2.5mm, 3mm), M4 stainless steel hardware

Connectivity

- Power: XT60 connector, MC4 solar connectors

- Internal: Various JST 2.4mm connectors (3-pin, 4-pin, 6-pin)

- Solar: 4mm solar cable, MC4 connector set

- Data Logging: Victron VE.Direct USB cable (MPPT to Raspberry Pi)

- Wiring: Various gauge wiring with ferrule connectors for secure connections

Key EMC Design Features

- Ground Isolation: Nylon standoffs provide electrical isolation

- Power Filtering: Separate DC-DC converters for different subsystems

- Signal Isolation: Ground loop isolator between codec and amplifier eliminates ground loops and hum

- Secure Connections: Ferrule connectors ensure reliable, low-resistance connections

- Shielding: Metal backplate (can be grounded for EMC improvement)

Operating Modes for EMC Testing

- Standby Mode: System powered, minimal activity

- WiFi Communication: Web interface access, scheduling updates

- BLE Communication: Mobile app connectivity, MPPT Bluetooth monitoring

- Audio Playback Mode: Full power audio output (scheduled bird calls)

- Bluetooth Audio Mode: Bluetooth audio streaming and playback

- PA Integration Mode: External PA system connectivity and control

- Data Logging Mode: Environmental sensor logging, power monitoring

- Recording Mode: Audio recording capability (microphone selection customer-configurable based on research requirements)

- Solar Charging: MPPT charging operation with monitoring

- Combined Mode: Simultaneous operations (WiFi + audio + charging + logging)

Power Specifications

- System Voltage: 12.8V nominal (LiFePO4 4S)

- Total System Current: Up to 5A peak (amplifier + processing)

- Solar Input: 30W panel via MPPT controller

- Battery Capacity: 6Ah (approximately 77Wh)

Frequency Allocations

- WiFi: 2.4GHz ISM band

- Bluetooth LE: 2.4GHz ISM band

- Switching Frequencies: DC-DC converters (to be specified from datasheets)

- Audio: 20Hz - 20kHz analog output

Notes for EMC Assessment

- Device uses pre-certified WiFi and BLE modules

- Class-D amplifier may generate high-frequency switching noise

- Multiple switching power supplies require EMC consideration

- Victron MPPT controller is professionally designed with established EMC compliance

- Ground loop isolator intentionally limits high-frequency response

- Recording mode available with customer-configurable microphone options (EMC assessment may vary based on microphone selection)

- Environmental sensor operates on I2C protocol (low EMC impact)